/*

Centre of Pressure (COP) Measurement System

For 4 half-bridge strain gauges with amplifiers

Arduino Mega 2560

*/

#include <LedControl.h>

// ==================== LED MATRIX PIN DEFINITIONS ====================

#define DIN_PIN  10

#define CS_PIN   11

#define CLK_PIN  13

// ==================== LED MATRIX INITIALIZATION ====================

LedControl lc = LedControl(DIN_PIN, CLK_PIN, CS_PIN, 1);

// ==================== SENSOR PIN DEFINITIONS ====================

const int BL_PIN = A0;

const int BR_PIN = A1;

const int FR_PIN = A2;

const int FL_PIN = A3;

// ==================== BASELINE OFFSET VARIABLES ====================

float baseline[4] = {0, 0, 0, 0};

// ==================== FILTER SETTINGS ====================

const int FILTER_SIZE = 10;

float filterBuffer[4][FILTER_SIZE];

int filterIndex = 0;

// ==================== TIMING ====================

const unsigned long SAMPLE_INTERVAL = 50;

unsigned long lastSampleTime = 0;

// ==================== THRESHOLD ====================

const float LOAD_THRESHOLD = 20;

bool loadDetected = false;

// ==================== TRACK LAST POSITION ====================

int lastRow = -1;

int lastCol = -1;

// ==================== DISPLAY 2x2 BLOCK ====================

void display2x2Block(int row, int col) {

// FIRST: Clear ALL rows to remove any previous LEDs

for (int r = 0; r < 8; r++) {

lc.setRow(0, r, 0x00);  // Clear each row

}

// Create pattern for 2x2 block

byte pattern = 0;

for (int c = 0; c < 2; c++) {

pattern |= (1 << (col + c));

}

// Write both rows at once

lc.setRow(0, row, pattern);

lc.setRow(0, row + 1, pattern);

}

// ==================== UPDATE LED POSITION ====================

void updateLEDPosition(int newRow, int newCol) {

// If position changed, update the display

if (newRow != lastRow || newCol != lastCol) {

display2x2Block(newRow, newCol);

lastRow = newRow;

lastCol = newCol;

}

}

// ==================== AUTO-ZERO ====================

void autoZeroSensors() {

const int SAMPLES = 100;

float sum[4] = {0, 0, 0, 0};

for (int i = 0; i < SAMPLES; i++) {

sum[0] += analogRead(BL_PIN);

sum[1] += analogRead(BR_PIN);

sum[2] += analogRead(FR_PIN);

sum[3] += analogRead(FL_PIN);

delay(5);

}

baseline[0] = sum[0] / SAMPLES;

baseline[1] = sum[1] / SAMPLES;

baseline[2] = sum[2] / SAMPLES;

baseline[3] = sum[3] / SAMPLES;

}

// ==================== SETUP ====================

void setup() {

Serial.begin(9600);

// Initialize LED Matrix

lc.shutdown(0, false);

lc.setIntensity(0, 8);

// Clear all rows initially

for (int r = 0; r < 8; r++) {

lc.setRow(0, r, 0x00);

}

// Auto-zero sensors

autoZeroSensors();

// Display center 2x2 block (rows 3-4, cols 3-4)

display2x2Block(3, 3);

lastRow = 3;

lastCol = 3;

Serial.println("System ready!");

}

// ==================== LOOP ====================

void loop() {

if (millis() - lastSampleTime >= SAMPLE_INTERVAL) {

lastSampleTime = millis();

// Read and filter sensors

float rawBL = readFiltered(0, BL_PIN);

float rawBR = readFiltered(1, BR_PIN);

float rawFR = readFiltered(2, FR_PIN);

float rawFL = readFiltered(3, FL_PIN);

// Subtract baseline

float BL = max(rawBL - baseline[0], 0);

float BR = max(rawBR - baseline[1], 0);

float FR = max(rawFR - baseline[2], 0);

float FL = max(rawFL - baseline[3], 0);

float total = BL + BR + FR + FL;

if (total > LOAD_THRESHOLD) {

loadDetected = true;

// Calculate COP

float COPx = ((FR + BR) - (FL + BL)) / total * 100.0;

float COPy = ((FL + FR) - (BL + BR)) / total * 100.0;

COPx = constrain(COPx, -100.0, 100.0);

COPy = constrain(COPy, -100.0, 100.0);

// Map COP to LED coordinates

int col = map(COPx, -100, 100, 0, 7);

int row = map(COPy, -100, 100, 0, 7);

col = constrain(col, 0, 7);

row = constrain(row, 0, 7);

// Keep 2x2 block within bounds

int startRow = constrain(row - 1, 0, 6);

int startCol = constrain(col - 1, 0, 6);

// Update LED position only if changed

updateLEDPosition(startRow, startCol);

// Print to Serial

Serial.print("Total: ");

Serial.print(total, 0);

Serial.print(" | COPx: ");

Serial.print(COPx, 1);

Serial.print("%, COPy: ");

Serial.print(COPy, 1);

Serial.println("%");

} else {

if (loadDetected) {

loadDetected = false;

// Return to center

updateLEDPosition(3, 3);

Serial.println("Center");

}

}

}

}

// ==================== MOVING AVERAGE FILTER ====================

float readFiltered(int sensorIndex, int pin) {

float newValue = analogRead(pin);

filterBuffer[sensorIndex][filterIndex] = newValue;

float sum = 0;

for (int i = 0; i < FILTER_SIZE; i++) {

sum += filterBuffer[sensorIndex][i];

}

filterIndex = (filterIndex + 1) % FILTER_SIZE;

return sum / FILTER_SIZE;

}